Align two or more images by optimizing the transformation parameters , such as scaling , rotation , and translation , to achieve the best match between them

import numpy as np

import matplotlib.pyplot as plt

# --- Image generation ---

def create_image(size=128):

img = np.zeros((size, size))

s1, s2 = size // 4, 3 * size // 4

img[s1:s2, s1:s2] = 1.0

return img

I_ref = create_image()

def transform_image(img, angle_deg=10, scale=1.1, tx=5, ty=-5):

angle = np.deg2rad(angle_deg)

h, w = img.shape

cy, cx = h / 2, w / 2

y, x = np.indices((h, w))

x = x - cx

y = y - cy

x_new = (x - tx) / scale

y_new = (y - ty) / scale

xr = np.cos(-angle) * x_new - np.sin(-angle) * y_new

yr = np.sin(-angle) * x_new + np.cos(-angle) * y_new

xr += cx

yr += cy

x0 = np.floor(xr).astype(int)

y0 = np.floor(yr).astype(int)

x1 = x0 + 1

y1 = y0 + 1

x0 = np.clip(x0, 0, w - 1)

x1 = np.clip(x1, 0, w - 1)

y0 = np.clip(y0, 0, h - 1)

y1 = np.clip(y1, 0, h - 1)

Ia = img[y0, x0]; Ib = img[y1, x0]; Ic = img[y0, x1]; Id = img[y1, x1]

wa = (x1 - xr) * (y1 - yr); wb = (x1 - xr) * (yr - y0)

wc = (xr - x0) * (y1 - yr); wd = (xr - x0) * (yr - y0)

warped = wa*Ia + wb*Ib + wc*Ic + wd*Id

return warped

# --- True transform ---

I_mov = transform_image(I_ref, angle_deg=12, scale=1.05, tx=8, ty=-5)

true_params = dict(angle_deg=12, scale=1.05, tx=8, ty=-5)

# --- Fitness ---

def fitness(I_ref, I_mov):

return -np.mean((I_ref - I_mov) ** 2)

# --- Cellular setup ---

n_cells = 100

cells = []

for _ in range(n_cells):

params = {

"angle_deg": np.random.uniform(-20, 20),

"scale": 1 + np.random.uniform(-0.2, 0.2),

"tx": np.random.uniform(-15, 15),

"ty": np.random.uniform(-15, 15),

}

warped = transform_image(I_mov, **params)

f = fitness(I_ref, warped)

cells.append({"params": params, "fitness": f})

# --- Optimization loop ---

n_iter = 150

alpha = 0.3

mutation_rate = 0.1

fitness_history = []

for it in range(n_iter):

new_cells = []

for i, cell in enumerate(cells):

neighbors = [cells[np.random.randint(0, n_cells)] for _ in range(2)]

best_neighbor = max(neighbors, key=lambda c: c["fitness"])

new_params = cell["params"].copy()

if best_neighbor["fitness"] > cell["fitness"]:

for key in new_params:

new_params[key] += alpha * (best_neighbor["params"][key] - cell["params"][key])

# small random mutation

if np.random.rand() < mutation_rate:

new_params["angle_deg"] += np.random.uniform(-1, 1)

new_params["tx"] += np.random.uniform(-0.5, 0.5)

new_params["ty"] += np.random.uniform(-0.5, 0.5)

new_params["scale"] *= (1 + np.random.uniform(-0.01, 0.01))

warped = transform_image(I_mov, **new_params)

f_new = fitness(I_ref, warped)

new_cells.append({"params": new_params, "fitness": f_new})

cells = new_cells

best = max(cells, key=lambda c: c["fitness"])

fitness_history.append(best["fitness"])

if it % 20 == 0:

print(f"Iter {it:03d} | Best fitness: {best['fitness']:.6f}")

# --- Show result ---

best = max(cells, key=lambda c: c["fitness"])

aligned = transform_image(I_mov, **best["params"])

plt.figure(figsize=(12, 4))

plt.subplot(1, 3, 1); plt.title("Reference"); plt.imshow(I_ref, cmap="gray"); plt.axis("off")

plt.subplot(1, 3, 2); plt.title("Moving"); plt.imshow(I_mov, cmap="gray"); plt.axis("off")

plt.subplot(1, 3, 3); plt.title("Aligned (Recovered)"); plt.imshow(aligned, cmap="gray"); plt.axis("off")

plt.figure(); plt.title("Fitness over iterations"); plt.plot(fitness_history); plt.xlabel("Iteration"); plt.ylabel("Best fitness"); plt.show()

print("\nTrue transform:", true_params)

print("\nRecovered transform:", best["params"])